1. Layer 1: Heterogeneous Nucleation Onset and Induction Lag

1.1 Dewpoint Temperature

Condensation begins when the surface temperature Ts falls below the dewpoint Tdp. The

dewpoint is computed from ambient temperature Tamb(℃) and relative humidity RH∈(0,1] via the August–Roche–Magnus approximation:

Tdp=blnRH+aTambb+Tamba-lnRH-aTambb+Tamb,  a=17.625, b=243.04℃                 (1)

The subcooling, the primary thermodynamic driver of condensation, is:

∆Tsub=Tdp-Ts,   ∆Tsub>0⟹condensation thermodynamically possible    (2)

1.2 Gibbs Free-Energy Barrier (Classical Nucleation Theory)

Before a condensate film appears, water-vapor molecules must surmount an energy barrier to form a stable nucleus. For heterogeneous nucleation on a substrate with contact angle θc, classical nucleation theory (CNT) gives:

∆G*=fθc∆Ghom*,   ∆Ghom*=16πσ33∆Gv2         (3)

Where σ is the liquid-vapor surface tension J m-3. The Fletcher heterogenous nucleation factor is:

fθc=2+cosθc1-cosθc24,    fθc∈0,1           (4)

Table 1 lists representative contact angles and their effect on the induction lag.

Table 1: Nominal contact angles and Fletcher factors for enclosure window coatings. The relative tind multiplier is with respect to untreated glass.

| Surface treatment | θc | fθc | Relative tind |
| --- | --- | --- | --- |
| Untreated glass | 20o | 0.006 | Baseline (rapid onset) |
| Hydrophilic coating | 10o | 0.001 | Shorter (faster nucleation) |
| Hydrophobic coating | 100o | 0.206 | ∼ 30× longer |
| Superhydrophobic | 150o | 0.933 | ∼ 150× longer |

1.3 Nucleation Rate

The nucleation rate per unit surface area is:

J∝exp-∆G*kBT ∝exp-fθcσ3kBT∆Tsub2                 (5)

Where the ∆Tsub-2 scaling comes from ∆Gv∝∆Tsub near saturation (Clausius-Clapeyron linearization).

1.4 Engineering Induction Lag

Inverting J and collecting all prefactors into the empirical coefficient kind gives the induction

lag:

tind=kindfθc∆Tsubnind                   (6)

2. Layer 2: Coverage Growth and Optical Attenuation

2.1 Logistic Coverage-Fraction Model

After tind, condensate droplets nucleate, grow by vapor diffusion, and coalesce. The fractional surface coverage C(t)∈0,1 during the growth phase is modelled as a logistic function:

Criset=Cmax1+exp-t-tind-t1/2wrise,  tind<t<theat        (7)

Where t1/2=1.5wrise centers the inflection point after onset, and the rise width scales with subcooling as:

wrise=w0∆Tsub                 (8)

2.2 Plateau Coverage Level

The maximum coverage Cmax is set by the balance between condensation driving force (∆Tsub) and available vapor supply (RH):

Cmax=Cmax,01-e-∆Tsub∆Tchar∙RHnRH      (9)

2.3 Optical Transmittance: Beer–Lambert Kernel

Coverage C(t) is mapped to transmittance τ(t) via the Beer-Lambert law:

τt=exp-αeff∙C(t),    αeff=-lnτmin                       (10)

Fig 1. τt  Model

Fig 2. Sensitivity sweep ∆T

References:

- Fletcher, N. H. (1958). Size effect in heterogeneous nucleation. Journal of Chemical Physics, 29(3), 572–576. https://doi.org/10.1063/1.1744540

- Carey, V. P. (2008). Liquid-vapor phase-change phenomena (2nd ed.). Taylor & Francis.

- Alduchov, O. A., & Eskridge, R. E. (1996). Improved Magnus Form Approximation of Saturation Vapor Pressure. Journal of Applied Meteorology and Climatology, 35(4), 601-609. https://doi.org/10.1175/1520-0450(1996)035<0601:IMFAOS>2.0.CO;2

- Thomas, T. M., Mahapatra, P. S., Ganguly, R., & Tiwari, M. K. (2023). Preferred mode of atmospheric water vapor condensation on nanoengineered surfaces: Dropwise or filmwise? Langmuir, 39(15), 5396–5407. https://doi.org/10.1021/acs.langmuir.3c00022

- Kim, H., Yang, Y., Kim, Y., Jang, D.-W., Choi, D., Park, K., Chung, S., & Kim, D. (2025). Effect of Droplet Contamination on Camera Lens Surfaces: Degradation of Image Quality and Object Detection Performance. Applied Sciences, 15(5), 2690. https://doi.org/10.3390/app15052690

- Zhang, K., Xu, Y., Xu, S., Lin, X., Zhou, J., Liu, Z., Li, Y., & Wei, H. (2025). Optical–Structural Optimization for Condensation Suppression in Automotive Camera Modules. Sensors, 25(21), 6515. https://doi.org/10.3390/s25216515

- Bohren, C. F., & Huffman, D. R. (1983). Absorption and scattering of light by small particles. Wiley.

- Hendrycks, D., & Dietterich, T. (2019). Benchmarking neural network robustness to common corruptions. Proceedings of the International Conference on Learning Representations (ICLR).

- Kong, X., et al. (2023). Benchmarking robustness of 3D object detection to common corruptions. arXiv. https://arxiv.org/abs/2303.11040