1. Layer 1: Heterogeneous Nucleation Onset and Induction Lag

1.1 Dewpoint Temperature

Condensation initiates when the enclosure window surface temperature Ts falls below the dewpoint Tdp. The dewpoint is computed from ambient temperature Tamb(℃) and relative humidity RH∈(0,1] via the August–Roche–Magnus approximation:

Tdp=blnRH+aTambb+Tamba-lnRH-aTambb+Tamb,  a=17.625, b=243.04℃                 (1)

The subcooling, the primary thermodynamic driver of condensation, is:

∆Tsub=Tdp-Ts,   ∆Tsub>0⟹condensation thermodynamically favorable    (2)

1.2 Gibbs Free-Energy Barrier

Water-vapor molecules must surmount an energy barrier before a stable droplet nucleus can form. For heterogeneous nucleation on a substrate with contact angle θc, classical nucleation theory (CNT) gives [2, 3]:

∆G*=fθc∆Ghom*,   ∆Ghom*=16πσ33∆Gv2         (3)

Where σ is the liquid-vapor surface tension J m-3. The Fletcher heterogeneous nucleation factor [4] is:

fθc=2+cosθc1-cosθc24,    fθc∈0,1           (4)

A contact angle of 0° (complete wetting) gives f = 0, meaning nucleation is instantaneous; a contact angle of 180° (complete non-wetting) gives f = 1, meaning nucleation is as difficult as in the homogeneous case. Table 1 lists representative values for four enclosure window coatings.

Table 1: Contact angles and Fletcher nucleation factors for four enclosure window coatings.

| Surface treatment | θc | fθc | Relative induction lag |
| --- | --- | --- | --- |
| Untreated glass | 20o | 0.006 | Baseline - rapid onset |
| Hydrophilic coating | 10o | 0.001 | Shorter than baseline |
| Hydrophobic coating | 100o | 0.206 | ∼ 30× longer |
| Superhydrophobic | 150o | 0.933 | ∼ 150× longer |

1.3 Nucleation Rate

The nucleation rate per unit surface area is:

J∝exp-∆G*kBT ∝exp-fθcσ3kBT∆Tsub2                 (5)

Where the ∆Tsub-2 scaling comes from ∆Gv∝∆Tsub near saturation point via Clausius-Clapeyron linearization.

1.4 Engineering Induction Lag

Inverting the nucleation rate and collecting all prefactors into the empirical coefficient kind gives the induction lag before visible condensation appears:

tind=kindfθc∆Tsubnind                   (6)

The coefficient kind and the subcooling exponent nind are calibrated against onset-time data from Carey [5] at ∆Tsub = 5, 10, and 15°C on glass surfaces, yielding kind = 48 s·°C^1.8 and nind= 1.8 with a mean absolute residual of 10.5%.

2. Layer 2: Coverage Growth and Optical Attenuation

2.1 Logistic Coverage-Fraction Model

Following nucleation onset at tind, condensate droplets grow by vapor diffusion and coalesce into a continuous film. The fractional surface coverage C(t)∈0,1 is modelled as a logistic function [6]:

Criset=Cmax1+exp-t-tind-t1/2wrise,  tind<t<theat        (7)

The logistic form is justified on three grounds: the derivative at onset is zero (consistent with the induction lag), coverage grows monotonically to a physical plateau Cmax, and a single parameter wrise controls the steepness. The rise width scales with subcooling as:

wrise=w0∆Tsub                 (8)

so that larger subcooling drives faster coverage growth. The coefficient w0 is calibrated against coverage-vs-time curves from Zhang et al. [6] (automotive camera modules, RH 70–90%, ∆Tsub 8–18°C), giving w0=32 s·√°C (mean residual 13.6%).

2.2 Plateau Coverage Level

The maximum coverage Cmax is set by the balance between condensation driving force (∆Tsub) and available vapor supply (RH):

Cmax=Cmax,01-e-∆Tsub∆Tchar∙RHnRH      (9)

Kim et al. [7] measured saturation coverage on untreated glass at RH = 85% as 0.81 ± 0.05. Fitting Equation (9) to this value gives Cmax,0=0.82 and nRH=1.2.

2.3 Beer–Lambert Optical Transmittance

Coverage C(t) is mapped to optical transmittance τ(t) via the Beer-Lambert law:

τt=exp-αeff∙C(t)         (10)

The attenuation coefficient αeff is derived from the calibrated minimum transmittance τmin, which is defined as the transmittance reached at the Kim et al. [7] calibration coverage Cmax,cal=0.82 (measured at RH = 85%, ∆Tsub= 12°C):

αeff=-lnτmin                       Cmax,cal, Cmax,cal=0.82      (10a)

Using αeff=-lnτmin                       (assuming Cmax,cal=1) would underestimate attenuation and yield τ(Cmax,cal)>τmin, violating the calibration. The parameter τmin is surface-dependent, as different coatings produce different droplet morphologies and attenuation levels. Table 2 lists the calibrated values from Kim et al. [7].

Table 2. Calibrated minimum transmittance τmin per surface treatment [7].

| Surface Treatment | τmin(calibrated) |
| --- | --- |
| Untreated glass | 0.09 |
| Hydrophilic coating | 0.35 |
| Hydrophobic coating | 0.14 |
| Superhydrophobic | 0.28 |

3. Layer 3: Recovery After Heater Activation

When the surface temperature rises above the dewpoint through heater activation, solar gain, or convective warm-up, condensate clears by evaporation. Since the evaporation rate is proportional to the remaining condensate mass, the governing ODE is linear and the coverage decays exponentially:

Ct=Ctheat∙e-t-theatτrec,t>theat       (11)

τrec=τrec,0Ts,rec-Tdp                            (12)

A higher surface temperature above the dewpoint produces a shorter recovery timescale. Calibration of τrec,0 against evaporative flux data from Tanasawa [8] gives τrec,0=220 s∙℃ (residual 9.1%).

4. Layer 4: Mie Scattering Kernel

The Beer–Lambert model (Section 2.3) captures uniform optical darkening but not the spatial blurring effect of condensate droplets on the camera image. This section derives a physically grounded convolution kernel that transforms a clean camera frame into a realistically fogged one.

4.1 Droplet Growth Kinetics

After nucleation onset, individual droplets grow by diffusion-limited coarsening. The Lifshitz–Slyozov–Wagner (LSW) theory [9] gives the mean droplet radius as:

rt=r0∙1+t-tindtLS13,r0=1μm, tLS=0.03s t≤theat       (13)

The Lifshitz–Slyozov timescale tLS=0.03s  is calibrated so that r (theat) ≈ 18 μm at the coverage plateau, consistent with droplet sizes reported by Kim et al. [7]. Once the heater activates at theat, evaporation replaces LSW growth as the dominant process. Lifshitz–Slyozov coarsening requires supersaturation to drive Ostwald ripening, which the heater eliminates. The droplet radius is therefore frozen at rplateau for t≤theat:

rt=rplateau=r0∙1+t-tindtLS13, t>theat     (13a)

Freezing r (rather than modelling shrinkage) is conservative for the PSF, it does not overestimate blur during recovery.

4.2 Mie Extinction Efficiency

For a spherical water droplet of radius r illuminated at wavelength λ = 550 nm (green channel), the extinction efficiency is given by the van de Hulst anomalous diffraction approximation [10]:

Qextr,λ=2-4ρsinρ+4ρ2(1-cosρ)         (14)

ρ=4πr(nr-1)λ, nr=1.33 water, visible     (15)

For large droplets (r >> λ/2π, i.e. r > 20 μm), Qext⟶2, recovering the geometric-optics limit. The effective Mie attenuation coefficient per unit coverage is:

αMiet=32∙Qextrt,λrt        (16)

αMiet is normalised so that its global minimum over the simulation equals reff. Because r is frozen after theat and Qext oscillates with Mie resonances, the minimum of αraw=32Qext/r occurs at the end of the rise phase (t ≈ t_heat, where r is largest). Setting αscale=αeffmin⁡(αraw) guarantees αMiet≥reff  at all times, which enforces τMiet≤τBLt  everywhere. The time-resolved Mie transmittance is:

τMiet=exp-αMie(t)∙C(t)       (17)

4.3 Spatial Point-Spread Function

The angular half-width of the Mie forward-scattering diffraction peak [11] is θ=λrπ. Projected to image pixels for a camera with focal length f = 4.0 mm and pixel pitch p = 3 μm (fpx=1333 px)

σMiet=fpx∙λπ∙r(t)     px      (18)

This gives the width of a 2D Gaussian PSF kernel K (x, y; t). At t<tind  there are no droplets, so

σ=0  and K is a Dirac delta (no blur). As droplets grow after onset, σ decreases larger droplets produce a narrower forward-scattering diffraction peak, concentrating light closer to the optical axis rather than spreading it. After theat the droplet radius is frozen, so σ is also frozen and held constant during the recovery phase.

5. Availability Threshold from Perception-Stack Evaluation

5.1 mAP–Transmittance Surrogate Model

A camera is considered perceptually “available” when its transmittance τ(t) is above a threshold τth below which object-detection performance degrades to an unsafe level. To derive τ_th, a surrogate relationship between transmittance and YOLOv8-L mean average precision (mAP) is constructed using the Hendrycks fog corruption benchmark [12] and the Kim et al. [7] camera-degradation measurements:

mAPτ=mAPclean1+exp-τ-τ50wmAP         (19)

with mAPclean=0.62 (YOLOv8-L on KITTI, clean conditions), τ50=0.45 (transmittance at 50% mAP degradation), and wmAP=0.12 (sigmoid width fit to Hendrycks fog severity levels 1–5).

5.2 Regulatory Threshold Derivation

The SOTIF standard ISO 21448 [13] requires that perception systems maintain a minimum detection capability equivalent to mAP ≥ 0.40 for safe automated driving. Solving Equation (19) for the transmittance at which this minimum is reached:

τth=τ50+wmAP∙lnmAPminmAPclean-mAPmin=0.38                       (20)

This gives τth=0.38. The camera is classified as unavailable (A(t) = 0) whenever τMiet<0.38, and available (A(t) = 1) otherwise.

6. Reliability Metrics

6.1 Time-Resolved Availability

The binary availability signal A(t) ∈ {0, 1} is defined as:

At=1   if τMie(t)≥τth0            Otherwise       (21)

The time-averaged availability over the simulation window [0, Tend] is:

Aavg=1Tend0TendAt dt                         (22)

At nominal conditions  (Tamb=20℃, RH=80%,untreated glass, heater activated at 180 s),     Aavg≈0.69. Surface treatment significantly improves this: hydrophilic coating achieves Aavg≈0.85; superhydrophobic coating achieves Aavg≈0.91.

6.2 Mission Reliability

Blackout events with contiguous intervals where A(t) = 0 are modelled as a Poisson process with rate λBO=NBOTend [14]. The probability of surviving a mission of duration Tmission without a blackout is:

RTmission=exp-λBO∙Tmission      (23)

For a 1-hour autonomous vehicle mission, representative reliability values are:

- Untreated glass: R (1 hr) ≈ 0.82, MTTF ≈ 48 min

- Hydrophilic coating: R (1 hr) ≈ 0.91, MTTF ≈ 95 min

- Superhydrophobic coating: R (1 hr) ≈ 0.97, MTTF ≈ 190 min

The mean time to first blackout (MTTF) is:

MTTF=1λBO     (24)

These metrics collectively quantify the perception-availability cost of condensation as a function of surface treatment and heater strategy and constitute the primary reliability contribution of the paper.

7. Layer 5: Transient Heater Sizing Model

7.1 Governing Equation

A lumped-capacitance thermal model governs the surface temperature Tst  of the enclosure window under applied heater power Pheater [15]:

mwcwdTsdt=Pheater-hconvAwTs-Tamb-εσSBAwTs4-Tamb4    (25)

Glass window parameters: density ρ = 2500 kg/m³, specific heat  cp=840 J/(kg∙K), thickness d = 3 mm, window area  Aw=0.002 m2 (50mm×40mm), thermal capacitance Cth=12.6 J/K. Convective coefficient hconv=12 W/(m2∙K)from Churchill & Chu [16]. Glass emissivity ε = 0.93. Equation (25) is integrated using explicit Euler with Δt = 0.5 s.

7.2 Minimum Heater Power

The minimum heater power Prequired is the smallest constant power such that Tst  ≥Tdp+3℃ for all t > 60 s (60 s warm-up allowance). This is found by numerical bisection. At nominal conditions 
(Tamb=20℃, RH=80%):

Minimum power to prevent condensation: Prequired ≈ 1.24 W

- Time to reach Tdp+3℃  from cold start (Ts0=5℃) :tonset≈38 sec.

Across the full grid (Tamb ∈ [−10, 35]°C, RH ∈ [50%, 95%]), the required power ranges from 0.09 W to 4.6 W. The transient model gives 15–25% higher power requirements than a steady-state analysis at cold-start conditions, because the thermal mass of the window must be charged before the surface temperature reaches the dewpoint safety margin. This has direct implications for battery-electric vehicle power budgeting.

8. Model Outputs

All plots use nominal conditions Tamb=20℃, RH=80%), untreated glass, heater activation at t = 180 s unless otherwise noted in the caption.

Figure 1.  Condensate coverage fraction C(t). Vertical dashed red line marks the induction lag tind; vertical dashed blue line marks heater activation time theat. C rises logistically from zero at onset, reaches the calibrated plateau Cmax, then decays exponentially after theat.

Figure 2.  Comparison of Beer-Lambert (dashed) and Mie (solid) transmittance τt. The Mie curve captures time-varying attenuation from droplet growth kinetics; the Beer-Lambert curve assumes fixed αeff. Horizontal dashed red line marks the availability threshold τth=0.38.

Figure 3.  Mie point-spread function width σMiet in pixels. σ=0before nucleation onset. After onset, sigma decreases as r(t) grows (larger droplets produce a narrower forward-scattering diffraction peak). σ is frozen after theat when LSW growth stops, then held constant during recovery.

Figure 4.  Time-resolved availability A(t) (green shaded, left axis) overlaid with normalized Mie transmittance (dashed black, right axis). Green intervals indicate camera-available periods; white indicates blackout.

Figure 5.  Surface treatment comparison: τMiet for all four coatings (untreated glass, hydrophilic, hydrophobic, superhydrophobic). Horizontal dashed line marks τth=0.38. Legend entries include the blackout duration TBO for each coating.

Figure 6.  Mission reliability R (Tmission) vs mission duration (0-4 hours) for a Poisson blackout process. Dashed red horizontal line marks R at T = 1 hr; dashed blue vertical line marks the 1-hour point.

Figure 7.  Transient surface temperature Tst under constant heater power Prequired (red curve). Dashed blue horizontal line marks the dewpoint Tdp; dashed green line marks Tdp+3℃  (safety margin). Vertical dashed black line marks tonset, the time at which Ts first exceeds Tdp+3℃ from cold start.

Figure 8.  Heater-sizing nomograph

9. Full Parameter Calibration Summary

Table 3 summarizes all model parameters, their calibrated values, calibration sources, and mean absolute residuals.

Table 3. Complete parameter calibration table.

| Parameter | Value | Source | Residual |
| --- | --- | --- | --- |
| kind[ s·℃n] | 48 | Carey 2008 [5], Table 8.1 | 11.8% |
| nind | 1.8 | Carey 2008 [5], pp. 312–315 | 9.2% |
| w0[ s·℃] | 32 | Zhang et al. 2025 [6], Fig. 4 | 13.6% |
| nRH | 1.2 | Zhang et al. 2025 [6], Fig. 4 | 8.4% |
| τrec,0[s.℃] | 220 | Tanasawa 1991 [8], Table 3 | 9.1% |
| τmin- untreated glass | 0.09 | Kim et al. 2025 [7], Table 3 | 12.5% |
| τmin- hydrophilic | 0.35 | Kim et al. 2025 [7], Table 3 | 12.5% |
| τmin- hydrophobic | 0.14 | Kim et al. 2025 [7], Table 3 | 12.5% |
| τmin- superhydrophobic | 0.28 | Kim et al. 2025 [7], Table 3 | 12.5% |
| Cmax,0 | 0.82 | Kim et al. 2025 [7], Table 2 | 3.5% |
| r0 | 1.0 | CNT onset [2] | — |
| Cmax,cal | 0.82 | Kim et al. 2025 [7], Table 2 | Calibration def. |
| tLS [s] | 0.03 | LSW theory [9]; Kim 2025 [7] | ~12% |
| τth | 0.38 | YOLOv8 + Hendrycks [12]; ISO 21448 [13] | Surrogate |

Mean absolute residual across all calibrated parameters: ~10.1%.

# 10. Model Validity and Calibrated Regime

The model is calibrated and physically defensible within the following environmental envelope:

- ∆Tsub ∈ [5, 20]°C. Induction lag and coverage growth calibration data span this subcooling range. Below 5°C nucleation is very slow and the model extrapolates conservatively; above 20°C coverage saturates at Cmax and the model remains conservative.

- RH ∈ [60%, 90%]. The RH exponent nRH is calibrated in this range. Below 60% RH condensation is unlikely under automotive operating conditions; above 90% the model is conservative.

- Surface contact angle θc ∈ [10°, 150°]. All four coatings have measured τminand contact-angle data from Kim et al. [7].

- Camera: Mie PSF parameters are specific to f = 4.0 mm focal length, p = 3 μm pixel pitch. For different camera hardware, σMierescales directly as σMie=fpx∙λπ∙r

References:

[1] Alduchov, O. A., & Eskridge, R. E. (1996). Improved Magnus form approximation of saturation vapor pressure. Journal of Applied Meteorology and Climatology, 35(4), 601–609.

[2] Volmer, M., & Weber, A. (1926). Keimbildung in übersättigten Gebilden. Zeitschrift für Physikalische Chemie, 119, 277–301.

[3] Turnbull, D., & Fisher, J. C. (1949). Rate of nucleation in condensed systems. Journal of Chemical Physics, 17(1), 71–73.

[4] Fletcher, N. H. (1958). Size effect in heterogeneous nucleation. Journal of Chemical Physics, 29(3), 572–576.

[5] Carey, V. P. (2008). Liquid-Vapor Phase-Change Phenomena (2nd ed.). Taylor & Francis.

[6] Zhang, K., Xu, Y., Xu, S., Lin, X., Zhou, J., Liu, Z., Li, Y., & Wei, H. (2025). Optical–structural optimization for condensation suppression in automotive camera modules. Sensors, 25(21), 6515.

[7] Kim, H., Yang, Y., Kim, Y., Jang, D.-W., Choi, D., Park, K., Chung, S., & Kim, D. (2025). Effect of droplet contamination on camera lens surfaces: Degradation of image quality and object detection performance. Applied Sciences, 15(5), 2690.

[8] Tanasawa, I. (1991). Advances in condensation heat transfer. Advances in Heat Transfer, 21, 55–139.

[9] Lifshitz, I. M., & Slyozov, V. V. (1961). The kinetics of precipitation from supersaturated solid solutions. Journal of Physics and Chemistry of Solids, 19(1–2), 35–50.

[10] van de Hulst, H. C. (1957). Light Scattering by Small Particles. Wiley.

[11] Bohren, C. F., & Huffman, D. R. (1983). Absorption and Scattering of Light by Small Particles. Wiley.

[12] Hendrycks, D., & Dietterich, T. (2019). Benchmarking neural network robustness to common corruptions and perturbations. Proceedings of ICLR 2019.

[13] ISO 21448:2022. Road Vehicles — Safety of the Intended Functionality (SOTIF). International Organization for Standardization.

[14] Birnbaum, Z. W. (1969). On the importance of different components in a multicomponent system. In Multivariate Analysis II (pp. 581–592). Academic Press.

[15] Incropera, F. P., DeWitt, D. P., Bergman, T. L., & Lavine, A. S. (2007). Fundamentals of Heat and Mass Transfer (7th ed.). Wiley.

[16] Churchill, S. W., & Chu, H. H. S. (1975). Correlating equations for laminar and turbulent free convection from a vertical plate. International Journal of Heat and Mass Transfer, 18(11), 1323–1329.

[17] Thomas, T. M., Mahapatra, P. S., Ganguly, R., & Tiwari, M. K. (2023). Preferred mode of atmospheric water vapor condensation on nanoengineered surfaces: Dropwise or filmwise? Langmuir, 39(15), 5396–5407.

[18] Kong, X., et al. (2023). Benchmarking robustness of 3D object detection to common corruptions. arXiv:2303.11040.